Brittas North Beach

Hippocampe Wheelchair First Use and Precautions for Use

Date

First Use:

- As is the case with classic wheelchairs, there is a period of adaptation for using the Hippocampe.

- The exterior all terrain wheels serve as a hand rim for self-propelling users, the grip inside the rim provides better handling.

- The balloon wheels require the assistance of a carer/personal assistant.

- The Hippocampe wheelchair is designed for a maximum weight of 130kg = 20.47 stone.

Precautions for Use:

- The Hippocampe wheelchair is not a boat. It allows entry into the water up to the height of the wheelchair axle and is not designed to float with the user in it.

- Do not immerse the Hippocampe wheelchair – water must not go above the height of the rear axle.

- Wearing a buoyancy aid is obligatory if entering the water. It is also obligatory to remain

within the designated swimming area noted by signs located on the beach.

- On entering water, it is recommended that you lean forward. (The buoyancy of the front wheel can throw you off balance backwards).

- A carer/personal assistant is required for entering the water.

- Users must be pushed/assisted exclusively by capable adults (especially when balloon wheels are in use).

- The utmost caution is recommended on beaches and steep slipways (apply breaks).

- Do not use the Hippocampe wheelchair when a wheel is punctured or flat.

- Using the Hippocampe wheelchair with more than one person in the seat is prohibited.

- Standing up on the seat or the footrest guard is prohibited.

- Modifying the structure of the wheelchair is prohibited.

- Avoid going under obstacles with the Hippocampe wheelchair.

- Avoid hand-braking on steep surfaces due to the risk of heating and injury.

- Do not move about on inclinations greater than 15%.

- Do not attempt to clear obstacles larger than 10cm.

- Do not smoke while using the wheelchair, the seat is designed not to catch fire (within 15 seconds) on contact with cigarettes or matches, allowing the user time to react.

- You must always be accompanied for entering water, or when the requirement of a harness is in use.

I have read/listened, understand and agree to abide by the terms of use of the Hippocampe wheelchair.

Mandatory

Signed	Printed	Mobile No.